# Zabbix administrator guide: Onboarding Nokia 7250 IXR-X1b with NETCONF

Audience: Zabbix administrators operating an existing Zabbix deployment who need to monitor Nokia 7250 IXR-X1b (SR Linux) using NETCONF, without relying on SNMP.
Goal: Enable the device, validate NETCONF, and collect metrics using Zabbix’s native SSH + NETCONF subsystem (7.2+), with a clear fallback path for older servers or bulk collectors.

| Platform | Nokia 7250 IXR-X1b |
| --- | --- |
| NOS | SR Linux |
| Primary protocol | NETCONF over SSH (subsystem netconf) |
| Primary Zabbix path | Native (Zabbix ≥ 7.2): SSH agent + ssh.run[...,netconf] |
| Fallback | External poller → trapper only if Zabbix < 7.2 (lab stack does not use this) |
| Secondary (optional) | gNMI (often already used by Nokia EDA / other collectors) |
| Out of scope | Replacing your Zabbix install; full DC fabric design |

## 1. Purpose and scope

This guide helps you:

- Use native NETCONF monitoring in Zabbix 7.2 and later (SSH item subsystem support).

- Prepare an X1b for model-driven monitoring.

- Onboard one X1b (or a small set) into an existing Zabbix server/proxy estate.

- Start with a practical metric set (reachability, identity, interfaces, optics presence) and extend later (DOM / DCO).

- Know when to fall back to an external poller (Zabbix < 7.2, heavy bulk inventory, or air-gapped jump hosts).

It is written for customer production or lab Zabbix, not as a requirement to install a particular container stack.

## 2. Why NETCONF for X1b (and not SNMP)

| Topic | SNMP | NETCONF |
| --- | --- | --- |
| Native Zabbix collection (7.2+) | SNMP agent items | SSH agent + subsystem netconf |
| Data contract | MIB / OID | YANG modules → hierarchical XML |
| Transport | Typically UDP/161 | SSH subsystem (SR Linux often TCP 22, not classic 830) |
| X1b / SR Linux fit | Often incomplete or operationally brittle | First-class model-driven read of config + operational state |
| Config vs state | Mixed OIDs | Explicit datastores / state leaves via <get> / filters |

Fair operational statement:
With NETCONF you can retrieve any operational or configuration node the NOS exposes in YANG for that session (subject to capabilities, features, and AAA)—not an arbitrary internal structure, and not a MIB.

## 3. How Zabbix does NETCONF (native, 7.2+)

### 3.1 What changed in Zabbix 7.2

Zabbix 7.2 extended SSH agent items with a sixth key parameter: subsystem.

ssh.run[<unique short description>,<ip>,<port>,<encoding>,<ssh options>,<subsystem>]

Setting subsystem to netconf opens the remote NETCONF SSH subsystem instead of a normal shell. The item’s Executed script field then carries NETCONF RPC XML, framed with the standard NETCONF end-of-message marker ]]>]]>.

Official documentation examples:

| Example | Meaning |
| --- | --- |
| ssh.run[Cisco1234,192.0.2.18,,,,netconf] | SSH/NETCONF to 192.0.2.18 (default port 22) |
| ssh.run[SFTPBackup,192.0.2.18,,,,sftp] | Same pattern for SFTP subsystem |

Reference: Zabbix SSH checks · What’s new in 7.2

Native support (important): Starting with Zabbix 7.2, NETCONF is a first-class, built-in monitoring path—not a third-party plugin and not dependent on an external poller. Zabbix did not add a new menu item named “NETCONF”; instead it extended the existing SSH agent item with a subsystem parameter. Operationally that is native NETCONF: the server/proxy opens the device’s netconf SSH subsystem and runs your RPCs from the Executed script field.

| If you read… | What it means |
| --- | --- |
| “Native NETCONF in Zabbix 7.2+” | Use SSH agent + key ssh.run[...,netconf] (this guide’s primary path) |
| “No separate NETCONF item type” | UI type is still SSH agent; capability is the 6th key parameter netconf |
| “External poller” | Optional fallback only for Zabbix < 7.2 or special topologies (§9)—not required on 7.2+ |

### 3.2 Recommended architecture (production, Zabbix ≥ 7.2)

┌────────────────────┐   SSH subsystem netconf    ┌──────────────────┐
│ Zabbix server or   │ ─────────────────────────► │ 7250 IXR-X1b     │
│ active proxy       │   (typically TCP 22)       │ SR Linux         │
│ SSH pollers        │                            └──────────────────┘
│ (StartSSH ≥ 1)     │
└─────────┬──────────┘
          │ raw XML reply → preprocessing
          │ (regex / JS → fields / JSON)
          ▼
┌────────────────────┐
│ Dependent items,   │
│ LLD, triggers, UI  │
└────────────────────┘

| Pros | Cons |
| --- | --- |
| No external poller process | Requires Zabbix ≥ 7.2 |
| Credentials and items live in Zabbix (macros / vault) | Large YANG trees need careful filters + JS preprocessing |
| Matches official pattern (e.g. Juniper MX by NETCONF templates) | Server/proxy must be built with libssh (preferred) or libssh2 |
| Works with proxies (poll near the devices) | Each master SSH item opens a session; plan intervals |

### 3.3 Fallback: external poller + trapper (any version)

Use when:

- Zabbix is still < 7.2, or

- You need heavy multi-RPC bulk collection with custom Python, or

- NETCONF must run from a jump host that is not a Zabbix proxy.

NETCONF poller (ncclient) ──► X1b
        │
        └── zabbix_sender / trapper ──► Zabbix server or proxy

This repository’s lab stack (repo root: docker-compose.yml, scripts/) uses the same native SSH+NETCONF path (Compose: Zabbix 7.4, ZBX_STARTSSH, zabbix_register_bleafs.py). The legacy poller script remains only as a reference for Zabbix < 7.2.

### 3.4 Pattern summary

| Pattern | Min Zabbix | When to use |
| --- | --- | --- |
| A. SSH agent + netconf subsystem | 7.2 | Default for production X1b onboarding |
| B. Poller → trapper | Any | Legacy Zabbix, jump hosts, complex bulk scripts |
| C. External check / Script item | Any | Few metrics; Python on server/proxy |
| D. SSH to CLI only | Any | Not model-driven; avoid for optics/state trees |

## 4. Prerequisites

### 4.1 Zabbix version and packages

| Requirement | Detail |
| --- | --- |
| Version | 7.2 or later for native NETCONF (this guide’s primary path) |
| SSH support | Server/proxy built/packaged with libssh (recommended) or libssh2 |
| Pollers | StartSSH ≥ 1 on the server or proxy that will dial the X1b (increase for many hosts) |
| Permissions | Create hosts, templates, items, macros, triggers |

Confirm version in the UI (Reports → System information or Administration) or zabbix_server -V.

### 4.2 Network

- ☐ Path from Zabbix server or proxy (the SSH poller) to the X1b management address.

- ☐ TCP 22 open for SSH/NETCONF (see §5—do not assume port 830).

- ☐ If using proxies, assign the host to the regional proxy so SSH originates in-region.

- ☐ DNS or static mapping for the Zabbix technical host name.

### 4.3 Device (X1b / SR Linux)

- ☐ Management IP and hostname known.

- ☐ SSH works with a monitoring identity.

- ☐ NETCONF server enabled and operational (§5).

- ☐ AAA allows that identity to perform NETCONF get (read-only preferred).

- ☐ Change control agreed if EDA/NMS owns the config.

### 4.4 Authentication secrets

| Method | Zabbix item fields |
| --- | --- |
| Password | Authentication method Password; username + password (prefer secret macros) |
| Public key | Authentication method Public key; configure Zabbix SSHKeyLocation and key files on the server/proxy |

## 5. Prepare the X1b: enable NETCONF (SR Linux)

### 5.1 Confirm platform

show version
# Expect SR Linux and chassis type consistent with 7250 IXR-X1b

### 5.2 Enable NETCONF

On SR Linux, NETCONF is commonly an SSH subsystem attached to an existing management ssh-server instance (often named mgmt), not a separate listener on port 830.

MD-CLI / candidate example:

enter candidate
system netconf-server mgmt {
    admin-state enable
    ssh-server mgmt
}
commit now

Verify:

info system netconf-server *
info from state system netconf-server *

Expect admin-state enable and oper-state up for the instance you configured.

### 5.3 Port and connectivity

| Expectation | Detail |
| --- | --- |
| Port | 22 when bound to ssh-server (lab and many SRL deployments) |
| Classic 830 | May be unused; do not assume 830 is open |
| Subsystem name | netconf (what Zabbix puts in the 6th key parameter) |

### 5.4 Production account (recommended)

| Practice | Guidance |
| --- | --- |
| User | Dedicated monitoring user (e.g. zabbix-ro) |
| Rights | Read-only / get operations; no edit-config if monitoring-only |
| Auth | Password or key per security policy; store secrets outside git |
| Source restriction | Limit SSH sources to Zabbix proxy/server IPs where possible |

### 5.5 Config ownership / EDA

If Nokia EDA or another controller reconciles configuration, NETCONF enablement must live in the source of truth. Otherwise monitoring may break after a reconcile.

## 6. Validate NETCONF before creating Zabbix objects

Run these from the same host/network path that will poll (Zabbix server or proxy).

### 6.1 Optional client probe (this repo)

python3 netconf_probe.py \
  --host <MGMT_IP_OR_FQDN> \
  --port 22 \
  --user <USER> \
  --password '<SECRET>' \
  --mode caps

Success: ok capabilities=<N> (N often hundreds of YANG modules on SRL).

python3 netconf_probe.py \
  --host <MGMT_IP_OR_FQDN> --port 22 \
  --user <USER> --password '<SECRET>' \
  --mode hostname

### 6.2 Optics inventory (optional)

python3 netconf_optics_inventory.py \
  --host <MGMT_IP_OR_FQDN> --port 22 \
  --user <USER> --password '<SECRET>'

On simulators, all cages may show not-present until real optics exist.

### 6.3 Filter tip (SR Linux)

Prefer bare subtree filters:

<interface><transceiver/></interface>

Avoid inventing YANG namespace URIs unless you know the exact URN—incorrect namespaces often return Unknown namespace.

## 7. Configure Zabbix — native SSH + NETCONF (primary)

### 7.1 Host design

| Field | Recommendation |
| --- | --- |
| Host name (technical) | Stable ID, e.g. site-a-bleaf-01.example.com |
| Visible name | Human label, e.g. Site A · BLeaf-01 · 7250 IXR-X1b |
| Groups | e.g. Nokia SR Linux, Border leaf, site/region |
| Interface | Agent (or other) interface with management IP—required by Zabbix even for SSH items |
| Proxy | Regional proxy if used |
| Tags | platform=7250-IXR-X1b, os=SR-Linux, monitor=netconf, role=bleaf |
| Macros | See §7.2 |

Do not attach SNMP interfaces for this path unless you intentionally dual-stack later.

### 7.2 Host / template macros (suggested)

| Macro | Example | Purpose |
| --- | --- | --- |
| {$NETCONF.IP} | 10.10.10.21 | Address used in the SSH item key (may equal interface IP) |
| {$NETCONF.PORT} | 22 | SSH/NETCONF port (22 for typical SRL) |
| {$NETCONF.USER} | zabbix-ro | Device username (item Username field) |
| {$NETCONF.PASSWORD} | (secret) | Item password; use secret macros / vault |
| {$NETCONF.TIMEOUT} | 30s | Item timeout (UI timeout field) |

Prefer secret macros where supported.

### 7.3 Master item: SSH agent + subsystem netconf

Create an item of type SSH agent:

| Field | Value |
| --- | --- |
| Name | e.g. NETCONF: Get system hostname |
| Type | SSH agent |
| Key | ssh.run[SrlHostname,{$NETCONF.IP},{$NETCONF.PORT},,,netconf] |
| Username | {$NETCONF.USER} |
| Authentication method | Password (or Public key) |
| Password | {$NETCONF.PASSWORD} |
| Executed script | See sample RPC below |
| Type of information | Text (or Log for very large replies) |
| Update interval | e.g. 1–5 minutes for identity; longer for heavy trees |
| Timeout | Sufficient for the device (e.g. 15–60s) |

Key rules (official):

- First parameter (unique short description) must be unique per host for each SSH item.

- Default port in the key is 22 if you leave port empty—set {$NETCONF.PORT} explicitly.

- Sixth parameter netconf selects the NETCONF subsystem.

### 7.4 Executed script: NETCONF RPC framing

For subsystem netconf, the Executed script is not a shell command. It is one or more RPC documents, each terminated by ]]>]]>.

Official multi-RPC shape (from Zabbix docs, Juniper-style RPC names shown as example):

<rpc>
  <get-software-information/>
</rpc>
]]>]]>
<rpc>
  <close-session/>
</rpc>
]]>]]>

SR Linux note: devices often advertise NETCONF 1.0 and 1.1. Prefer a client hello that announces only base:1.0 so message framing stays ]]>]]> (1.1 uses chunked framing). Always end with <close-session/>.

SR Linux-oriented baseline — hostname (operational state):

<?xml version="1.0" encoding="UTF-8"?>
<hello xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">
  <capabilities>
    <!-- capability URI (not the XML namespace URI) -->
    <capability>urn:ietf:params:netconf:base:1.0</capability>
  </capabilities>
</hello>
]]>]]>
<rpc xmlns="urn:ietf:params:xml:ns:netconf:base:1.0" message-id="1">
  <get>
    <filter type="subtree">
      <system xmlns="urn:nokia.com:srlinux:general:system">
        <name xmlns="urn:nokia.com:srlinux:chassis:system-name"/>
      </system>
    </filter>
  </get>
</rpc>
]]>]]>
<rpc xmlns="urn:ietf:params:xml:ns:netconf:base:1.0" message-id="2">
  <close-session/>
</rpc>
]]>]]>

Version / platform (adapt filter to the leaves you need):

<rpc message-id="1">
  <get>
    <filter type="subtree">
      <system>
        <information/>
      </system>
    </filter>
  </get>
</rpc>
]]>]]>
<rpc message-id="2">
  <close-session/>
</rpc>
]]>]]>

Transceiver / optics presence (all interfaces with a transceiver container):

<rpc message-id="1">
  <get>
    <filter type="subtree">
      <interface>
        <transceiver/>
      </interface>
    </filter>
  </get>
</rpc>
]]>]]>
<rpc message-id="2">
  <close-session/>
</rpc>
]]>]]>

Notes:

- Always end sessions with <close-session/> when the NOS expects a clean close (mirrors official examples).

- Keep filters narrow—unfiltered full-device <get> can be multi‑megabyte.

- Reply size is limited (Zabbix documents up to 16MB for the SSH item return; DB limits also apply).

- Exact YANG paths can vary by SR Linux release; validate filters with a client probe first (§6).

### 7.5 Preprocessing pattern (master + dependent items)

Zabbix returns the raw NETCONF XML (or multi-reply stream) as the master item value. Extract metrics with preprocessing, then dependent items / LLD.

Recommended pattern (same idea as official Juniper MX by NETCONF templates):

- Master SSH item — one RPC (or a small bulk set); Type = Text.

- Preprocessing on master (optional):

  - JavaScript to normalize XML → JSON, strip framing, or count tags.

  - Or leave raw XML and parse per dependent item.

- Dependent items — JSONPath / regex / JS to pull single values (hostname, oper-state counts, etc.).

- LLD rule (later) — JS that emits discovery JSON for per-port optics.

Example: extract hostname with regex preprocessing (adjust to actual XML):

| Step | Type | Parameters |
| --- | --- | --- |
| 1 | Regular expression | Pattern: <name>([^<]+)</name> · Output: \1 |
| 2 | Trim | (optional) |

Example: count not-present optics with JavaScript (illustrative):

// value = raw XML string from master item
var n = (value.match(/not-present/g) || []).length;
return n;

Example: availability — use a simple check or treat successful master item history as up:

| Item | Type | Idea |
| --- | --- | --- |
| net.tcp.service[ssh,{$NETCONF.IP},{$NETCONF.PORT}] | Simple check | Port open |
| Master SSH item not unsupported | Built-in | Session + RPC succeeded |

### 7.6 Suggested starter item set

| Purpose | Approach | Notes |
| --- | --- | --- |
| NETCONF TCP up | Simple check net.tcp.service[ssh,...] | Fast fail for ACL/routing |
| Hostname | SSH master + regex/JS | Identity / inventory drift |
| Software version | SSH master + preprocess | Change detection |
| Interface summary | SSH master + JS | Keep filter bounded |
| Optics cages / present / missing | SSH master (transceiver filter) + JS counts | Tier A presence |
| Per-port DOM / DCO | Master + LLD + dependent prototypes | After real optics |

Template name suggestion: Template Nokia SR Linux NETCONF (SSH).

### 7.7 Triggers (initial)

| Name | Idea | Severity |
| --- | --- | --- |
| NETCONF TCP down | net.tcp.service[...] = 0 for N minutes | High |
| NETCONF item unsupported | Item becomes unsupported (auth, RPC error, timeout) | High / Average |
| Hostname mismatch | last hostname ≠ expected inventory | Warning |
| Optics present drop | drop in present count (real hardware only) | Average / High |

Suppress noisy “all cages empty” alerts on simulators.

### 7.8 Templates and scale

- Create the template with macros and SSH master items.

- Add dependent items and triggers on the template.

- Link to each X1b host; override macros per host.

- Export YAML/XML into GitOps / backup.

- Study official Juniper MX by NETCONF as a structural reference (SSH masters + JS preprocess + dependent items + LLD)—vendor RPCs differ, pattern does not.

### 7.9 UI verification

- Open Monitoring → Latest data for the host.

- Confirm master SSH items update and are not Unsupported.

- Confirm dependent fields (hostname, counts) populate.

- Use Test on the item (where available) while developing RPCs.

## 8. End-to-end checklist: onboard one X1b (native path)

### Phase A — Plan

- Confirm Zabbix ≥ 7.2 on the server/proxy that will poll.

- Record mgmt IP, hostname, site, proxy.

- Confirm StartSSH and libssh packaging.

- Agree monitoring user and secret handling.

### Phase B — Device

- SSH to the X1b; confirm SR Linux / X1b.

- Enable netconf-server bound to mgmt SSH (§5).

- Verify oper-state up.

- Persist config in automation source of truth if applicable.

### Phase C — Path validation

- From Zabbix server/proxy: ssh <user>@<mgmt_ip>.

- Optional: capability + hostname probes (§6).

- Confirm TCP 22 from that host.

### Phase D — Zabbix objects

- Create host group(s), host, tags, interface IP, proxy.

- Set {$NETCONF.*} macros.

- Link Template Nokia SR Linux NETCONF (SSH) or create SSH items (§7).

- Add simple-check reachability + triggers.

### Phase E — Accept

- Latest data shows XML/extracted values; items supported.

- Enable agreed triggers.

- Document host in CMDB; note NETCONF owner.

- Hand off to NOC with dashboard link.

### Phase F — Next X1b

- Enable NETCONF → macros → template link → verify Latest data.

## 9. Fallback: external poller + trapper (Zabbix < 7.2 or bulk)

### 9.1 When to use

- Server still on 7.0 LTS (or earlier) without subsystem support.

- Need multi-host Python orchestration, complex parsing, or offline inventory dumps.

- NETCONF can only be initiated from a non-proxy jump host.

### 9.2 Item keys (trapper)

| Key | Value type | Description |
| --- | --- | --- |
| netconf.availability | Numeric (unsigned) | 1 = probe succeeded |
| netconf.caps | Text | Capability summary |
| netconf.hostname | Text | System host-name |
| netconf.if.summary | Text | Interface oper summary |
| netconf.version | Text | Software version |
| netconf.optics.cages | Numeric | Transceiver containers |
| netconf.optics.present | Numeric | Installed optics |
| netconf.optics.not_present | Numeric | Empty cages |
| netconf.optics.summary | Text | Histogram / debug |

Trapper items require exact technical host name and key. Include clock timestamps when using JSON sender protocol.

### 9.3 Poller behaviour (reference)

scripts/netconf_poller.py connects with ncclient, collects metrics, and sends via trapper. Production needs process supervision, backoff, and secret injection.

### 9.4 External check alternative

Script on server/proxy: item type External check, key e.g. netconf_get.sh[{$NETCONF.IP},{$NETCONF.USER},{$NETCONF.PASSWORD},hostname]. Heavier per-item load than bulk SSH masters with dependents.

## 10. Optics monitoring guidance

### 10.1 What NETCONF can provide (YANG)

| Tier | Content | When available |
| --- | --- | --- |
| A – Presence | oper-state, oper-down-reason, tx-laser, fault flags | Empty cages included |
| B – Inventory | vendor, PN, SN, form-factor, PMD, wavelength | Optic installed |
| C – DOM | temperature, voltage, Rx/Tx power, thresholds | Optic + DDM |
| D – DCO | frequency, OSNR, BER, CD, DGD (srl_nokia-interfaces-dco) | Coherent / ZR-class |

### 10.2 Practical rollout

- Tier A via native SSH master + JS counts (or poller).

- Tier B inventory strings as dependent items.

- Tier C/D numeric items + thresholds when fiber/coherent is live.

- Prefer LLD for per-port items once cardinality is known.

## 11. Troubleshooting

| Symptom | Checks |
| --- | --- |
| Item type SSH missing / unsupported key | Upgrade to ≥ 7.2; confirm package includes SSH support |
| SSH item unsupported: library | Server/proxy linked with libssh/libssh2 |
| Cannot connect | Routing, ACL, credentials, mgmt network-instance; try from proxy host |
| Works on 830 elsewhere, fails on X1b | Use port 22; confirm netconf-server + ssh-server |
| Auth fails | Username/password macros; key files under SSHKeyLocation |
| Empty or error RPC reply | Filter namespaces; simplify subtree; test with netconf_probe.py |
| Unknown namespace | Bare filters; no guessed YANG URNs |
| Huge values / timeout | Narrow filter; increase item timeout; reduce interval concurrency |
| Dependent items empty | Master value shape changed; fix regex/JS/JSONPath; use preprocessing test |
| Caps work in Python, Zabbix fails | Subsystem spelling netconf; RPC framing ]]>]]>; unique key description |
| Worked then stopped | Config reconcile removed NETCONF; password rotation; ACL change |
| Poller trapper processed: 0 | Wrong technical host name; item not trapper type; key typo |

### Useful device commands

info from state system netconf-server *
info system ssh-server *
show version

### Useful Zabbix checks

- Server/proxy log for SSH poller errors

- Monitoring → Queue (SSH backlog)

- Item configuration: type SSH agent, 6th key param netconf

- Macro resolution on the host

- StartSSH in zabbix_server.conf / proxy conf

## 12. Security

- Prefer read-only device accounts for monitoring.

- Prefer secret macros or vault integration for passwords.

- Restrict SSH sources to Zabbix proxy/server addresses.

- Protect private keys used by the Zabbix process.

- Rotate device and Zabbix API credentials per policy.

- Do not commit production secrets to repositories.

- Lab sample passwords in any demo materials are not for production.

## 13. Extending beyond the baseline

| Goal | Approach |
| --- | --- |
| Many X1bs | Template + API/Ansible host create + macro fill |
| Per-port optics | LLD from JS preprocessing of transceiver XML |
| Numeric DOM / DCO | Dependent float items + triggers |
| Dashboards | Filter tags monitor=netconf, platform=7250-IXR-X1b |
| gNMI | Separate collector; optional trapper or other pipeline |
| Still on Zabbix 7.0 | Use §9 poller→trapper until upgrade to 7.2+ |

## 14. Quick reference

Platform:     Nokia 7250 IXR-X1b / SR Linux
Native:       Zabbix ≥ 7.2 — NETCONF via SSH subsystem (built-in; no external poller)
UI type:      SSH agent  (not a separate "NETCONF" type name)
Key:          ssh.run[<unique>,{$NETCONF.IP},{$NETCONF.PORT},,,netconf]
Script:       client hello base:1.0 + <rpc>...</rpc> + ]]>]]> + close-session
Port:         22 (typical when netconf-server bound to ssh-server mgmt)
Enable:       system netconf-server <name> { admin-state enable; ssh-server <ssh>; }
Parse:        Preprocessing + dependent items (regex / JS / JSONPath)
Fallback:     ncclient poller → trapper only if Zabbix < 7.2
Optics:       Start with presence; add DOM/DCO when pluggables exist
UI check:     Monitoring → Latest data · items not Unsupported
Docs:         zabbix.com SSH checks · What’s new 7.2 (subsystem)

## 15. Reference materials (optional lab)

Helpers in this repository for demos and script reuse—not required when using native SSH+NETCONF on a customer Zabbix 7.2+ estate:

| Path | Role |
| --- | --- |
| README.md | Lab stack quick start (native SSH NETCONF) |
| docker-compose.yml | Zabbix 7.4 + Postgres; ZBX_STARTSSH |
| scripts/zabbix_register_bleafs.py | API: hosts + ssh.run[...,netconf] + dependents |
| scripts/netconf_probe.py | Optional capability / hostname probes |
| scripts/netconf_optics_inventory.py | Optional optics inventory |
| scripts/netconf_poller.py | Legacy trapper poller (fallback only) |
| Zabbix-Lab-NETCONF-Overview.pptx | Slide overview of the lab |

Official Zabbix references:

- SSH agent items (subsystem / NETCONF examples)

- What’s new in Zabbix 7.2 — NETCONF via SSH subsystem

- Template pattern: Juniper MX by NETCONF (SSH masters + preprocessing + dependents)

## Document control

| Title | Zabbix administrator guide: Onboarding Nokia 7250 IXR-X1b with NETCONF |
| --- | --- |
| Primary method | Zabbix ≥ 7.2 SSH agent + subsystem netconf |
| Intended use | Customer Zabbix operations / professional services runbook |
| Related lab | Magic Kingdom containerlab (optional; native SSH NETCONF) |
| Revision note | Lab stack migrated from poller→trapper to native ssh.run[...,netconf] (Zabbix 7.4, StartSSH, SRL client hello + YANG namespaces, ZBX_TIMEOUT) |